CENICA MÉXICO

VEHICLE EMISSIONS EVALUATION

Results Delivery

| Date | 22-Sept-05 | Type of Test | 505-HOT |
| --- | --- | --- | --- |
| Organism | Bio Plus Fuel | Additive/Apparatus | NO |
| Vehicle Type | VW | Catalitic Converter | NO |
| Year Model | 1995 Sedan | Combustible | Gasoline |
| No. of licence |  | Inertial mass (lb) | 2000 |
| Motor (liters) | 1.6 | Road cargo (hp) | 77 (@ 50 mph) |
| Transmission | MANUAL | Equipment operator | Tec. Victor Duarte |
| Vehicular weight (kgs) | 820 | Vehicle operator | Tec. Melquiades Alcantara |
| P. B. V. |  | Observations |  |
| Odometer |  |  |  |

PARAMETERS

Cycle 505

| Barometric pressure (mmHg) | 584 |
| --- | --- |
| Temperature (°C) | 21 |
| Vapor pressure (mmHg) | 15.65 |
| Revolutions | 10690 |
| Mix Vol. | 2434.8 |
| Distance (km) | 5717 |
| Relative Humidity (%) | 47 |
| Factor DT | 17.4 |

|  | LECTURES | PARTIAL RESULTS |
| --- | --- | --- |

|  |  | Sample | Room |  | (grams) | (g/km) |
| --- | --- | --- | --- | --- | --- | --- |
| CO |  | 1309 | 51 |  | 138.72 | 24.25 |
| CO2 |  | 78 | 2 |  | 750.9 | 131.36 |
| HC |  | 1283 | 77 |  | 14.45 | 2.53 |
| NOx |  | 610 | 21 |  | 7.48 | 1.3 |

FINAL RESULTS

| CO |  | CO2 |  | HC |  | NOX |  | PERF. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| g/Km |  | g/Km |  | g/Km |  | g/Km |  | Km/l |
| 24.26 |  | 131.36 |  | 2.53 |  | 1.31 |  | 13.49 |

Done by				Verified by				Vo. Bo.

Tec. Melquiades Alcatar Báez	      Ing. Alejandro García Fragoso

Lab. de Emisiones Vehiculares	      Evaluación de Emisiones Vehiculares